# Junos OS (JNCIA Study Notes)

## Configuration Basics

Command: show configuration
Displays the active configuration in a hierarchical format.

### Major Configuration Hierarchies

- system – Contains all system-wide settings such as hostname, users, authentication, NTP, DNS, and services (SSH, NETCONF, etc.).

- chassis – Contains physical device settings, such as hardware-related options, alarms, and redundancy features. (Note: On some platforms, this hierarchy is limited.)

- interfaces – Contains all interface-related configuration, including physical interfaces, logical units, IP addressing, VLAN tagging, and encapsulation.

- protocols – Contains routing and switching protocol configuration (e.g., OSPF, BGP, RIP, IS-IS, STP).

## Configuration Management

- Junos OS stores up to 49 previous committed configurations (configuration rollback files).

- Configuration changes are not applied immediately. Changes are first made in the candidate configuration and only take effect after a commit.

### Commit Confirm (Safety Feature)

- Junos provides a commit confirm mechanism.

- If you commit changes with commit confirm, the system will automatically roll back to the previous configuration if the commit is not confirmed within the specified time (default is 10 minutes, not one minute).

- This feature is especially useful when making remote changes that could cause loss of connectivity.

## Management Interfaces

### CLI (Command-Line Interface)

- Junos OS uses a hierarchical CLI, which is consistent across all Juniper devices.

- The CLI supports operational mode and configuration mode.

### J-Web

- J-Web is Juniper’s web-based graphical user interface (GUI).

- It allows administrators to monitor device status and perform configuration changes through a browser.

- J-Web is typically used for basic configuration and monitoring rather than advanced or large-scale deployments.

## Automation and Scripting

### Juniper PyEZ

- Juniper PyEZ is a Python library used to automate and manage Junos devices.

- It communicates with devices using NETCONF.

- PyEZ allows tasks such as configuration changes, operational command execution, and retrieval of device information through Python scripts.

- Commonly used in network automation and DevOps workflows.

### On-Box Scripts

- On-box scripts run directly on the Junos device.

- They are written in languages supported by the device, such as SLAX, XSLT, or Python (platform-dependent).

- Used for event handling, configuration validation, and automation that must execute locally on the device.

### Off-Box Scripts

- Off-box scripts run on an external system (such as a server or workstation).

- They interact with Junos devices remotely using protocols like NETCONF, SSH, or APIs (e.g., via PyEZ).

- Commonly used for centralized automation, orchestration, and large-scale network management.

## Hardware Architecture (JNCIA Relevant)

### Fixed-Port Chassis

A fixed-port chassis is a network device where the number and type of interfaces are permanently built into the hardware.

- Interfaces cannot be added or removed via modules

- Common in access-layer switches and branch routers

- Typically lower cost and simpler to deploy

- Examples include many EX-series switches and SRX branch devices

### Modular Chassis

A modular chassis is a network device that supports removable hardware components such as line cards, Routing Engines, and power supplies.

- Interfaces are provided through interchangeable line cards

- Designed for scalability and high availability

- Common in data center and service provider environments

- Examples include MX-series routers and high-end EX switches

### Routing Engine (RE)

The Routing Engine (RE) is the control plane component of a Junos device.

- Runs Junos OS

- Handles routing protocols (OSPF, BGP, IS-IS, etc.)

- Maintains the routing table (RIB) and programs the forwarding table (FIB)

- Provides management services (CLI, NETCONF, SNMP)

- In high-availability platforms, multiple Routing Engines may be present for redundancy

## Juniper Device Families (JNCIA Overview)

### MX Series Routers

MX routers are high-performance Ethernet routers primarily used in service provider and large enterprise networks.

- Designed for WAN, edge, and core routing

- Support high throughput and advanced routing features

- Use modular and fixed platforms depending on model

- Commonly run BGP, OSPF, IS-IS, and MPLS

### PTX Series Routers

PTX routers are ultra-high-capacity core routers designed for large-scale service provider and cloud networks.

- Optimized for packet transport and IP/MPLS core roles

- Focus on extremely high bandwidth and scalability

- Typically used in backbone and data center interconnects

- Limited feature set compared to MX (intentionally simplified for scale)

### ACX Series Routers

ACX routers are access routers used at the network edge.

- Commonly deployed in metro Ethernet and service provider access networks

- Support aggregation of customer traffic

- Smaller form factor compared to MX and PTX

- Often used with MPLS and Ethernet services

### SRX Series Firewalls

SRX devices are Juniper’s next-generation firewalls.

- Provide stateful firewalling, NAT, and security services

- Can function as both firewalls and routers

- Used in branch, data center, and perimeter security roles

- Run Junos OS with security-focused features

## Accessing the Device

SSH Access:
Use the following command from a terminal application to access a Junos device remotely:

ssh username@ip_address

This connects you to the device’s CLI in operational mode by default.

## CLI Modes

### Operational Mode

Operational mode is used to monitor, troubleshoot, and manage the device without changing configuration.

- Default mode after login

- Prompt ends with >

- Safe mode (no configuration changes allowed)

Common operational mode command groups:

- show – Displays system information and status (e.g., interfaces, routes, configuration)

  - Show system uptime

  - Show system information

  - Show lldp neighbors

- monitor – Provides real-time output (e.g., traffic, log messages)

- clear – Clears counters, statistics, or sessions

- request – Performs operational actions such as system reboot or software install

### Configuration Mode

Configuration mode is used to make changes to the device configuration.

- Entered by typing configure from operational mode

- Prompt ends with #

- Changes are made to the candidate configuration

- Changes do not take effect until a commit is issued

## Junos OS Release Types (JNCIA Exam Topic)

Junos OS versions follow a structured release naming convention. Understanding these is important for the JNCIA exam.

### Feature Release (R1)

A Feature Release (R1) is the first release in a Junos software train.

- Introduces new features and enhancements

- May include new hardware support

- Considered less stable than later releases

- Typically not recommended for production unless new features are required

Example: 21.4R1

### Maintenance Release (R2, R3, Rx)

A Maintenance Release (MR) focuses on improving stability and fixing bugs found in earlier releases.

- No new major features are added

- Improves reliability and performance

- Most commonly deployed in production environments

- Higher number (R3, R4, etc.) generally means more fixes

Example: 21.4R3

### Service Release (S1, S2, Sx)

A Service Release (SR) is a specialized release that addresses specific critical issues.

- Contains targeted bug fixes or security patches

- Usually recommended only when affected by a specific issue

- Applied on top of an existing release

Example: 21.4R3-S2

## Junos Interface Naming Convention (JNCIA Exam Topic)

Junos interface names follow a consistent format that identifies the interface type and its physical location in the device.

### Interface Type Prefixes

- ge – Gigabit Ethernet (1 Gbps)

- xe – 10-Gigabit Ethernet (10 Gbps)

- et – Ethernet interfaces operating at 40 Gbps and above (e.g., 40G, 100G)

### Interface Numbering Format

type-fpc/pic/port

Example:

ge-0/0/1

- type – Interface type (ge, xe, et)

- fpc – Flexible PIC Concentrator number

- pic – Physical Interface Card number

- port – Port number on the PIC

### Flexible PIC Concentrator (FPC)

An FPC (Flexible PIC Concentrator) is a major hardware module in modular Juniper devices.

- Houses one or more PICs

- Installed directly into the chassis

- Provides forwarding resources and connectivity

- Commonly referred to as a “line card”

### Physical Interface Card (PIC)

A PIC (Physical Interface Card) is a hardware module that provides the actual physical network ports.

- Installed into an FPC

- Determines port type and speed (e.g., GE, 10GE, 100GE)

- May support multiple ports per PIC

### Verifying Chassis Hardware

Use the following operational mode commands to view hardware details:

- show chassis hardware – Displays all installed hardware components

- show chassis fpc – Displays FPC status and details

- show chassis pic fpc-slot fpc-number – Displays PICs installed in a specific FPC