## Interface-Related Operational Commands

### Command Additional Subcommands

show interfaces terse
Displays a concise, one-line summary for each interface. This includes the interface name, administrative status (up/down), operational status (up/down), and assigned IP address (if any). This command is commonly used for quick health checks and troubleshooting.

show interfaces brief
Provides a brief summary of interfaces, similar to terse but with slightly more readable formatting. It shows interface status, protocol family, and addressing information. Useful for a quick overview without excessive detail.

show interfaces detail
Displays detailed operational information for each interface, including statistics such as packet counts, errors, drops, and interface configuration details. This command is useful when troubleshooting performance or error-related issues.

show interfaces extensive
Provides the most detailed output available for interfaces. In addition to everything shown in detail, it includes extensive statistics, queue information, protocol-specific data, and historical counters. This command is typically used for deep troubleshooting and advanced analysis.

## Protocol Families

inet
Represents the IPv4 protocol family in Junos OS. It is used on logical interfaces to configure IPv4 addresses and related Layer 3 settings. IPv4 addresses are configured under family inet.

inet6
Represents the IPv6 protocol family in Junos OS. It is used to configure IPv6 addresses and related settings under family inet6.

## Interface Configuration Principles in Junos

With Junos OS, physical properties are always configured on the physical interface.

Physical interface properties include:

- Physical MTU

- Speed

- Duplex

Logical properties are always configured on the logical interface (unit).

Logical interface properties include:

- Layer 3 MTU

- IP addresses

- VLAN tags

## Filtering and Matching Interface Output

show interfaces terse xe-0/1/*
Filters the output to display only interfaces that match the specified pattern.

“*” (asterisk wildcard)
In computing and in Junos CLI commands, the asterisk (*) is a wildcard character. It matches zero or more characters in a string. When used in Junos operational commands, it allows you to match multiple interfaces without typing each one explicitly.

Example:
xe-0/1/* matches all interfaces that start with xe-0/1/, such as:

- xe-0/1/0

- xe-0/1/1

- xe-0/1/2

This is commonly used for faster troubleshooting and bulk interface checks.

## Additional Interface Commands

show interfaces descriptions
Displays interface names along with their configured descriptions and current operational status. This command is useful for identifying interface purpose, connected devices, or links based on administrator-defined descriptions.

## Junos CLI Help, Piping, and Matching

show interfaces terse | ?
In Junos OS, the pipe symbol (|) is used to filter or process command output. Typing | ? after a command displays all available pipe operations that can be applied to that command’s output.

### “|” (pipe) — Definition in Junos

The pipe symbol (|) takes the output of one command and sends it to a filter or modifier. This allows you to narrow down, search, or format output without scrolling through large amounts of data.

Common pipe options include:

- match — Filters output to lines that match a specific string

- except — Excludes lines that match a string

- count — Counts matching lines

- find — Displays output starting from a match

- compare — Compares command output with a previous version

Key point (JNCIA): You can pipe multiple operations together in a single command.

Example:
show interfaces terse | match up | count

### “?” (question mark) — Definition in Junos

The question mark (?) is used for context-sensitive help in the Junos CLI. It shows available commands, keywords, or options based on where you are in the command hierarchy.

Uses include:

- Discovering valid commands

- Viewing available options without memorization

- Learning syntax interactively

Example:
show interfaces ?

### match up/down — Definition

The match pipe filters output to only display lines containing the specified text.

Examples:
show interfaces terse | match up
Displays only interfaces that are operationally or administratively up.

show interfaces terse | match down
Displays only interfaces that are down.

### Matching Physical vs Logical Interfaces

You can use match to distinguish between physical interfaces and logical interfaces.

Examples:
show interfaces terse | match ge-
Matches physical Gigabit Ethernet interfaces.

show interfaces terse | match ge-0/0/0.
Matches logical units (note the dot . indicating a logical interface).

### Case Sensitivity

Junos CLI matching is not case sensitive by default. This means:

- match UP

- match up

- match Up

All produce the same result.

JNCIA Exam Tip:
Understanding |, ?, and match is essential for efficient troubleshooting and is frequently tested conceptually on the JNCIA-Junos exam.

## Configuration Views in Junos OS

### Hierarchy View — Definition

Hierarchy view is the default configuration display format in Junos OS. It shows the configuration in a tree-like structure, reflecting how configuration statements are logically organized.

Command:
show configuration

This view makes it easy to understand the relationship between configuration elements, such as how system, interfaces, and protocols are structured.

JNCIA key point: Junos internally stores the configuration in hierarchy view.

### Set View — Definition

Set view displays the configuration as a series of individual set commands. Each line represents a complete configuration statement.

Command:
show configuration | display set

Set view is especially useful for: - Learning Junos configuration syntax - Copying and pasting configuration lines - Comparing configurations between devices

## Filtering Configuration Output

You can filter configuration output to view only specific sections.

Examples:

show configuration system | display set
Displays only system-related configuration in set format.

show configuration interfaces | display set
Displays only interface-related configuration.

show configuration protocols | display set
Displays only protocol-related configuration.

## Configuration Hierarchy Levels

Top-level hierarchy elements commonly tested on JNCIA include:

- system — Device-wide settings (hostname, users, services, logging)

- interfaces — Physical and logical interface configuration

- protocols — Routing and protocol configuration (OSPF, RIP, BGP, etc.)

These hierarchy levels also contain sub-level configuration statements.

Example:
set system syslog user * any emergency

## Basic System Configuration Examples

### Setting the Hostname

Command:
set system host-name R1

- R1 is an example hostname

### Enabling Management Services

Enable SSH:
set system services ssh

Enable Telnet:
set system services telnet

JNCIA note: SSH is recommended over Telnet because it is encrypted.

## User Configuration in Junos

### Creating a Local User (Set View)

Example:
set system login user netadmin class super-user authentication plain-text-password

You will be prompted to enter and confirm the password.

### User Classes in Junos

Junos uses user classes to define permission levels.

Common classes:

- super-user
Full administrative access. Equivalent to root-level privileges.

- operator
Limited access, typically used for monitoring and basic operational commands.

- read-only
Can view configuration and operational status but cannot make changes.

- unauthorized
No permissions. Rarely used.

JNCIA Exam Tip: Most administrative users are assigned the super-user class.

### Example: Creating a Read-Only User

set system login user viewer class read-only authentication plain-text-password

This user can view configuration and operational data but cannot modify the system.