# JNCIA-Junos (JN0-105) Notes

## Routing Protocols

Routing Protocols are protocols used by routers to exchange routing information and determine the best path to forward traffic across a network.

## Interior Gateway Protocols (IGP)

Interior Gateway Protocols (IGPs) are routing protocols used to exchange routing information within a single autonomous system (AS).

### Distance Vector Protocols

Distance Vector Protocols determine the best path based on distance (metric) and direction (next hop), sharing routing information with directly connected neighbors.

- RIP (Routing Information Protocol):

  - A distance vector routing protocol that uses hop count as its metric.

  - Maximum hop count is 15 (16 is considered unreachable).

  - Simple to configure but not scalable for large networks.

### Link-State Protocols

Link-State Protocols build a complete map of the network topology by exchanging link-state information with all routers in the area.

- Metric:

  - A value used by routing protocols to determine the best path.

  - Lower metric values are preferred.

- OSPF (Open Shortest Path First):

  - A link-state routing protocol that uses cost as its metric.

  - Cost is typically based on bandwidth.

  - Supports hierarchical design using areas.

  - Fast convergence.

  - Route preference (Junos): 10.

- IS-IS (Intermediate System to Intermediate System):

  - A link-state routing protocol commonly used by service providers.

  - Uses cost as its metric.

  - Scales well in large networks.

  - Uses levels (Level 1 and Level 2) instead of areas.

## Exterior Gateway Protocols (EGP)

Exterior Gateway Protocols (EGPs) are used to exchange routing information between different autonomous systems.

- BGP (Border Gateway Protocol):

  - The primary EGP used on the internet.

  - Uses path attributes instead of traditional metrics.

  - Focuses on policy-based routing rather than shortest path.

## OSPF Configuration (Junos)

Basic OSPF configuration example:

set protocols ospf area 0 interface xe-0/1/5.0

### OSPF Hello Packets

OSPF Hello packets are used to discover neighbors, maintain neighbor relationships, and elect a Designated Router (DR) and Backup DR on multi-access networks.

- OSPF Multicast Address:

  - 224.0.0.5 — All OSPF routers

  - 224.0.0.6 — All Designated Routers (DR/BDR)

- OSPF Multicast Addresses (IPv6):

  - ff02::5 — All OSPF routers

  - ff02::6 — All Designated Routers (DR/BDR)

### Neighbor Verification

Common operational commands:

- show ospf neighbor — Displays OSPF neighbor state information

- show route protocol ospf — Displays routes learned via OSPF

### OSPF Passive Interfaces

A passive interface participates in OSPF but does not send or receive OSPF hello packets. This is commonly used on interfaces connected to end-user networks.

Configuration example:

set protocols ospf area 0 interface xe-0/1/5.0 passive

## Routing Policy

Routing Policy controls which routes are accepted, modified, or advertised by a routing protocol. In Junos, routing policy is implemented using policy statements and can influence attributes such as route preference, metric, and acceptance or rejection of routes.

## OSPFv2 vs OSPFv3

OSPFv2 is used for IPv4 routing and carries IPv4 addressing information directly within OSPF packets.

OSPFv3 is designed for IPv6 routing and separates routing logic from IP addressing. It uses IPv6 link-local addresses and supports improved extensibility compared to OSPFv2.

- Junos CLI note:

  - OSPFv2 uses protocols ospf

  - OSPFv3 uses protocols ospf3

## Juniper Ethernet Switching Overview

### EX Series Switches

EX Series switches are Juniper Ethernet switches designed primarily for enterprise campus and branch networks.

- Common use cases:

  - Access and distribution layers

  - Campus LAN environments

- Example:

  - EX4300 — A high-performance access switch that supports Layer 2 and Layer 3 features.

### QFX Series Switches

QFX Series switches are designed for data center and high-performance environments.

- Common use cases:

  - Data centers

  - Spine-and-leaf architectures

- Optimized for high throughput and low latency.

## VLANs (Virtual LANs)

VLANs logically segment a Layer 2 network into separate broadcast domains.

### Verifying VLANs

- show vlans — Displays configured VLANs

- show ethernet-switching interface — Shows interface status and VLAN membership

### Viewing VLAN Configuration

- show configuration vlans

- show configuration vlans | display set — Displays VLAN configuration in set format

### Creating a VLAN

set vlans Guest vlan-id 30

### Configuring an Access Port

Set interface description:

set interfaces ge-0/0/1 description "NAME"

Configure access mode and assign VLAN:

set interfaces ge-0/0/1 unit 0 family ethernet-switching interface-mode access
set interfaces ge-0/0/1 unit 0 family ethernet-switching vlan members Corporate

### Verifying Interface Status

- show interfaces terse — Shows interface link status

- show ethernet-switching interfaces — Confirms access/trunk mode and VLAN membership

## MAC Address Table

The MAC address table displays Layer 2 forwarding entries learned by the switch.

### Viewing the MAC Table

- show ethernet-switching table — Displays all learned MAC addresses

- show ethernet-switching table vlan <vlan-name> — Displays MAC addresses for a specific VLAN

### MAC Table Flags

MAC table flags indicate how a MAC address was learned:

- Dynamic — Learned automatically from traffic

- Static — Manually configured entry

## Trunk Ports (802.1Q)

A trunk port carries traffic for multiple VLANs between switches or networking devices using IEEE 802.1Q tagging.

### Configuring a Trunk Port

set interfaces xe-0/2/0 unit 0 family ethernet-switching interface-mode trunk

### Adding VLANs to a Trunk

Add a single VLAN:

set interfaces xe-0/2/0 unit 0 family ethernet-switching vlan members VLAN_NAME

Add multiple VLANs:

set interfaces xe-0/2/0 unit 0 family ethernet-switching vlan members [ VLAN1 VLAN2 VLAN3 ]

## Router-on-a-Stick (802.1Q Subinterfaces)

In Junos, this configuration represents router-on-a-stick, not a traditional switch-based SVI.

Router-on-a-stick uses a single physical interface with multiple logical units (subinterfaces), each tagged for a different VLAN, to perform inter-VLAN routing.

- Commonly used on routers or Layer 3 devices acting as routers

- Requires 802.1Q VLAN tagging

- Each VLAN is mapped to a logical unit with its own IP address

### Enabling VLAN Tagging

set interfaces xe-0/1/0 vlan-tagging

### VLAN Tagging Modes

VLAN tagging modes define how VLAN tags are handled on an interface in Junos.

- VLAN Tagging (``):

  - Enables single VLAN tag (802.1Q) support.

  - Commonly used for router-on-a-stick designs.

  - Each logical unit is associated with one VLAN ID.

- Flexible VLAN Tagging (``):

  - Allows multiple VLAN tags and more advanced encapsulations.

  - Required for Q-in-Q (802.1ad) and complex VLAN scenarios.

  - Uses encapsulation types such as extended-vlan-bridge or vlan-bridge.

- Stacked VLAN Tagging (Q-in-Q):

  - Uses two VLAN tags (outer and inner).

  - Common in service provider networks to separate customer VLANs.

  - Requires flexible-vlan-tagging to be enabled.

### Creating Logical Units (Subinterfaces)

Assign VLAN ID:

set interfaces xe-0/1/0 unit 10 vlan-id 10

Configure IPv4 address:

set interfaces xe-0/1/0 unit 10 family inet address 172.16.10.1/24

Configure IPv6 address:

set interfaces xe-0/1/0 unit 10 family inet6 address 2001:db8:0:20::1/64

JNCIA exam note:

- Junos does not use the Cisco term SVI

- This design is best described as router-on-a-stick using logical units

### Verifying Router-on-a-Stick Configuration

Confirm interface and logical unit status:

show interfaces terse xe-0/1/0

Verify IP addressing on logical units:

show interfaces xe-0/1/0 extensive

Confirm routing table entries:

show route

Verification goal:

- Physical interface is up

- Logical units are up/up

- Correct IP addresses are assigned per VLAN

Include what i need to know about Mist AI and Marvin for the exam.

## Juniper Mist AI & Marvis (Exam Focus)

Mist AI is Juniper’s cloud-based, AI-driven networking platform designed to simplify operations and improve user experience across wired, wireless, and WAN environments.

### Key Mist Concepts for JNCIA

- Cloud-managed architecture:

  - Management and analytics are performed in the Mist cloud.

  - Devices maintain local forwarding even if cloud connectivity is lost.

- AI-Driven Operations (AIOps):

  - Uses machine learning to detect anomalies, predict issues, and optimize performance.

  - Focuses on user experience rather than just device status.

- Microservices architecture:

  - Allows rapid feature updates without disrupting network operations.

### Marvis Virtual Network Assistant

Marvis is the AI-driven virtual network assistant within Mist.

- Provides natural-language query capability (ask questions about the network)

- Performs root cause analysis and provides actionable recommendations

- Reduces troubleshooting time by correlating events across the network

### Exam-Relevant Takeaways 💡

- Mist = cloud-managed + AI-driven

- Marvis = virtual assistant for troubleshooting and insights

- Focus is on simplifying operations and improving user experience, not manual CLI management

- JNCIA expects conceptual understanding, not configuration